## Preventative Maintenance for Drum Fillers

Scale Base:

 : Check accuracy with known weights. (Re-calibration may be required)

 : Overall condition of scale

Lance Assembly:

Visual

 : Inspection of lance carriage rails

 : Inspection of limit switches on lance assembly, check for proper position & tightness

 : Wiper ring

 : Inspect and adjust cam followers

 : Wiper ring

 : Recalibrate lance and ensure encoder is working and counting properly

 : Foot Valve seat and rod packing (check for leaks) …Repack if needed

 : Drip pan

  : Piping from valve to foot valve (check for leaks)

  : Foot Valve Actuator

(Required only on Sub-surface Fillers)

:  Air motor leaks at fittings or valves.

 : Ball Screw (lubricate w/ lithium grease NLGI #2)

Structural:

: Check for loose bolts and screws

: Check alignment of chains, sprockets, and gears

: Check valve actuator mounting and operation of ball valve

: Check all air lines for leaks at fittings (cylinder, switches, operator enclosure, etc...)

: Check all air tubing for cracks or abrasion

Filter \Regulator

: Check for leaks

: Remove filter and replace if necessary

: If oiler is attached, check and maintain level w/ (SAE#10 non-detergent oil)

 : *Check operating range during fill cycle.  Required: 70-80 PSI

Enclosures

: Check Air Lines for leaks "check tubing for cracks or abrasion".

: Check for moisture or oil in enclosures

: Visually inspect electronics and connections

: Check enclosure seals

: Check all air lines for leaks at fittings (cylinder, switches, operator enclosure, etc...)

Conveyors

: Condition of rollers and chains

: Check turn tables and transfer cars for operation

: Check all gear boxes for fluid and report leaks

: Check and ensure photo eyes are operational and are aligned

: Check all alignment guides

CUSTOMER : ______________________________________

DATE:__________________________       INSPECTED BY:_______________________

Note: If necessary, make copies and utilize this sheet. Also please place notes and needed repairs below.